# Lesson plan 1: Logic gates

# About the lesson

This lesson is designed to give the students a basic understanding of how digital devices process data. This will enable them to appreciate why binary was adopted as the system for encoding data for computer processing.

Students will learn that the central processing unit, or CPU, of a computer is constructed from an enormous number of basic building blocks known as logic gates and will learn how combinations of these gates can make the decisions that underpin the processing that takes place within computers and other digital devices.

Learning objectives

By the end of this lesson, students will:

- know that digital devices rely on switches to make the decisions involved in data processing

- understand the operation of AND, OR and NOT logic gates

- be able to complete truth tables for logic gates

- understand the significance of 1s and 0s in data processing within digital systems.

National curriculum

The following subject content from the KS3 National Curriculum is included in this lesson:

- Understand simple Boolean logic (for example, AND, OR and NOT) and some of its uses in circuits and programming

- Understand how numbers can be represented in binary, and be able to carry out simple operations on binary numbers (for example, binary addition, and conversion between binary and decimal)

Links

Students who have completed the Programming in Scratch module will have encountered and used AND, OR and NOT conditional operators. In the Advanced spreadsheets module students will also have encountered the use of AND and OR. These concepts are also useful for the Python programming project in this course as well as in any further programming activity, computing courses or courses including digital control systems and logic.

Key terminology

input, output, switch, logic gate, AND gate, OR gate, truth table, NOT gate, inverter

Teaching and learning activities

Share the objectives for the lesson with students (Slide 2 of Lesson 1 presentation).

Starter

Introduce students to the animal sorting gate and a basic process in computing (Slides 3–6 and page 124).

Ask: What is the input for our animal sorting gate and what is the output? After a brief discussion, reveal the answer and make the link to binary and logic gates (Slides 7–12 and page 125).

Ask students to complete Activity 1.1: Animal sorting gate (Slide 13).

Discuss the activity as a class and ask the following questions to facilitate the activity.

- The animal is not a big cat, so is the first input 1 or 0? 0

- The animal has stripes, so is the second input 1 or 0? 1

- If the first input is 0 and the second input is 1, what is the output to the gate? If the inputs are 0 AND 1, the output is 0.

- Why does the gate remain closed? Because the output is 0 and the state of the gate is closed if the output is 0.

Part 1

Explain to students that, to understand how digital devices process data, it is essential to know a little bit about how the electrical circuits in the central processing unit of a computer, the CPU, carry out the processing.

Ask students to work in pairs to complete Activity 1.2: Circuits (page 126, Worksheet 1.2, slide 14).

Discuss how the circuits in Activity 1.2 work and explain that the CPU at the heart of a computer is built from millions of logic gates (Slides 15–17 and page 126).

Part 2

Explain that the logic gates in a computer's CPU are represented by standard symbols and introduce students to the AND gate symbol and the OR gate symbol (Slides 18–19 and page 126).

Introduce students to truth tables (Slide 20 and page 127).

Ask students to complete Activity 1.3: Truth tables (page 127, Worksheet 1.3, slide 21).

Answers

1.

| A | B | Out |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

2.

| A | B | Out |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

Part 3

Introduce students to the NOT gate (Slide 22 and page 126).

Ask students to complete Activity 1.4: Street light (page 127, Worksheet 1.4, slide 23).

Point out to students that quite complex decisions are made by the control system for the street light, yet all the data processed by the control system is in the form of binary digits, 1s and 0s.

Answers

1.

| In | Out |
| --- | --- |
| 0 | 1 |
| 1 | 0 |

2.

| Timer | Light sensor | Street light |
| --- | --- | --- |
| 1 | 1 | 1 |
| 0 | 1 | 0 |
| 1 | 0 | 1 |
| 0 | 0 | 1 |

Plenary

Check students' knowledge and understanding with a short question and answer session linked to the learning objectives for the lesson.

- In what form is data input into logic gates?

(Answer: Data is input as 1s and 0s.)

- In what form is the output from logic gates?

(Answer: Data is output as 1s and 0s.)

- What do we call a table that is used to summarise the possible inputs to and outputs from a logic gate?

(Answer: a truth table)

- If the inputs to an AND gate are 1 and 0, what is the output?

(Answer: 0)

- If the inputs to an OR gate are 1 and 0, what is the output?

(Answer: 1)

- What is the name of the gate that inverts the input?

(Answer: a NOT gate)

Homework

Ask students to complete Quiz 1 (slide 24).

Stretch activities

Some students will be able to appreciate the concept of logic gates quite quickly and should be encouraged to identify other situations using the same concepts.

Support activities

If available, physical electronic logic kits to demonstrate the concepts will be useful.

The tasks are suited to a paired approach and the choice of partner can provide essential support for those who find the activities and concepts more challenging.